Upgrading 4600 firmware
1. Download the Honeywell Scanning and Mobility (HSM) USB serial driver from the website www.honeywellaidc.com  (Go to Downloads > Software > Choose 4600g from the product list)

2. Extract All Files and Save them to a folder location on the PC.
3. Install the EZconfig utility.
4. Download and save the 4600 Standard Release Software from the website. 
5. Scan the ‘USB COM Port Emulation’ bar code from below. 
6. Open EZconfig > Go to File > Preferences > change to the COM port assigned to the 4600. 

7. Click on Download in the Application Explorer > Browse for the 4600 Standard Release Software under Firmware Download. 

8. Click ‘Download to Device’ > you should receive notification when the firmware is successfully installed. 

[image: image1.wmf]~

T

E

R

M

I

D

1

3

0

.

³

USB COM Port Emulation


